FA8 / FA10 SDK API User Guide

FA8API-SUNTEK.jar · com.suntek.SUNTEK

The API described in this document corresponds to the FA8/FA10 series products. You need to check the hardware specifications to confirm the actual peripherals supported by the board.

0. Integrate the new JAR (changes relative to the old document)

Application projects should only depend on the new package and should not include YNHAPI-20251224.jar at the same time.

| project | New JAR |
| --- | --- |
| JAR file | FA8API-SUNTEK.jar |
| Java package name | com.suntek |
| Entry Class | SUNTEK |
| Take an example | SUNTEK mAPI = SUNTEK.getInstance(); |
| Internal type | SUNTEK.GpioState, etc. |
| IP Entity | com.suntek.entity.IpConfig |
| Stored information | com.suntek.StorageInfo |

0.1 is placed in the project.

Copy FA8API-SUNTEK.jar to the app/libs/ directory of your Android project, and add the following to app/build.gradle:

dependencies { 
implementation files('libs/FA8API-SUNTEK.jar') 
}

0.2 Retrieve Interface Instance

All subsequent examples will use this mAPI.

import com.suntek.SUNTEK; 
import com.suntek.StorageInfo; 
import com.suntek.entity.IpConfig; 

SUNTEK mAPI = SUNTEK.getInstance();

0.3 Real Device Self-Test Entry

The relevant interfaces can be verified in the device's "Settings → User Settings". The current firmware menu text may still display YNHCommonAPI, which is the name of the pre-installed application, not the JAR class name.

The new test APK project is FA8CommonApi, and the source code package name is com.suntek.fa8.

Edit History

| date | Modification points |
| --- | --- |
| 20220531 | The single parameter `installApkSilently` has been deprecated; now, it automatically launches after installation by package name/class name. |
| 20220625 | Add serial number, device model, Ethernet MAC address, and IMEI. |
| 20220630 | Add Ethernet switch and status |
| 20220701 | Add App Protection |
| 20220713 | Add boot image/animation and drop-down status bar |
| 20220830 | Get screen on/off status |
| 20220922 | Multiple modes added to timed on/off |
| 20221107 | Add foreground protection to the app |
| 20230413 | Individual control of screen display switch |
| 20240321 | Modify Wiegand asynchronous read documentation |
| 20240521 | Add setInstallPackagePolicy to the whitelist for installation. |
| 20250310 | Add language settings |
| 20251224 | Increase watchdog timeout time |
| 20260813 | The document has been changed to Word; all examples have been switched to FA8API-SUNTEK.jar / com.suntek.SUNTEK |

2. System Parameters

Most setters require a subsequent call to mAPI.reboot() to take effect.

2.1 Obtain the motherboard model

String model = mAPI.getBoardModel();

2.2 Serial Number

2.2.1 Obtaining

String sn = mAPI.getSerialNo(); // Consistent with Build.SERIAL

2.2.2 Settings

String serialNo = "12345678"; 
mAPI.setSerialNo(serialNo); 
mAPI.reboot();

2.3 Ethernet MAC

String mac = mAPI.getEthernetMAC(); 
mAPI.setEthernetMAC("00:E0:EC:37:2A:42"); 
mAPI.reboot();

2.4 4G Module IMEI

String imei = mAPI.getIMEI(); 
mAPI.setIMEI("864793030776509"); 
mAPI.reboot();

2.5 Equipment Model

String model = mAPI.getProductModel(); // Consistent with Build.MODEL 
mAPI.setProductModel("123456"); 
mAPI.reboot();

2.6 Device Storage Information

Returns a list of com.suntek.StorageInfo locations, including internal storage, TF/SD, and USB storage. Unit: KB.

List<StorageInfo> list = mAPI.getStorageInfos(); 
for (StorageInfo info : list) { 
int type = info.getType(); // TYPE_MEMORY / TYPE_LOCAL_STORAGE / TYPE_TFCARD / TYPE_USB_STORAGE 
long total = info.getTotalSize(); 
long free = info.getFreeSize(); 
String path = info.getPath(); 
}

2.7 Boot Image

mAPI.setBootLogo("/sdcard/boot.bmp"); // Must be an 8-bit BMP

2.8 Boot Animation

mAPI.setBootAnimation("/sdcard/bootanimation.zip"); // Standard Android bootanimation

2.9 APK Installation Strategy (Whitelist)

SUNTEK.InstallPackagePolicy policy = mAPI.new InstallPackagePolicy(); 
List<String> allow = new ArrayList<>(); 
allow.add("com.example.app"); 
policy.setAllowInstallPackageList(allow); 
// ALLOW_INSTALL_TYPE_NORMAL = -1 Cancel policy 
// ALLOW_INSTALL_TYPE_NOT_ALLOW = 0 Whitelist only, otherwise reject 
// ALLOW_INSTALL_TYPE_PASSWORD = 1 Password required for non-whitelisted users 
policy.setAllowInstallType(SUNTEK.InstallPackagePolicy.ALLOW_INSTALL_TYPE_PASSWORD); 
policy.setAllowInstallPassword("1234abcd"); 
mAPI.setInstallPackagePolicy(policy);

2.10 Setting the Language

mAPI.updateLanguage("zh", "CN"); // Corresponds to values-zh-rCN

Commonly used: zh/CN, zh/TW, zh/HK, en/US, en/GB, ja/JP, ko/KR, etc., consistent with Android values-xx-rYY.

3. OTA upgrade

The upgrade package must be placed in the sdcard directory, and the file name should be update.zip.

mAPI.otaUpdate("/sdcard/update.zip");

4. APK Management

4.1 Automatic startup upon system boot

mAPI.setBootLaunchApk("com.example.app", true);

4.2 Silent Installation

`mAPI.installApkSilently("/sdcard/API.apk", "com.example", "com.example.MainActivity"); ` // The last two parameters being null indicates that 
`mAPI.installApkSilently("/sdcard/API.apk", null, null);` indicates that the application will not launch automatically after installation .

4.3 Silent Uninstallation

mAPI.uninstallApkSilently("com.example.app");

4.4 Application Guardian

Restarting does not retain the changes; the application needs to access them again after booting up.

// Non-foreground guardian, checks every 10 seconds. 
mAPI.setAppKeepLive("com.example.app", 10); 
mAPI.setAppKeepLive("", 1); // Cancel 

// Foreground guardian 
mAPI.setForegroundAppKeepLive("com.example.app", 10); 
mAPI.setForegroundAppKeepLive("", 1);

5. Display

5.1 / 5.2 Screen Rotation

SUNTEK.RotationDegree deg = mAPI.getScreenRotation(SUNTEK.ScreenType.MAIN); 
if (deg == SUNTEK.RotationDegree.DEGREE_90) { } 
mAPI.setScreenRotation(SUNTEK.ScreenType.AUX, SUNTEK.RotationDegree.DEGREE_180);

5.3 / 5.4 screen density

SUNTEK.LcdDensity density = mAPI.getLcdDensity(); 
mAPI.setLcdDensity(SUNTEK.LcdDensity.DENSITY_240);

5.5 / 5.6 Screen On/Off

isScreenOn() cannot be used to determine this function after sleep/wakeup; it is only applicable when the app itself calls setScreenOnOff.

boolean on = mAPI.isScreenOn(); 
mAPI.setScreenOnOff(false);

6. Watchdog

mAPI.enableWatchdog(true); 
mAPI.setWatchdogTimeout(15); // seconds, may vary by 1-2 seconds depending on the platform. 
mAPI.feedWatchdog(); // It is recommended to feed the watchdog approximately every 10 seconds. 
mAPI.enableWatchdog(false);

7. Shutting down and restarting

mAPI.shutdown(); 
mAPI.reboot();

8. Hibernation and Wake-up

PowerManager pm = (PowerManager) context.getSystemService(Context.POWER_SERVICE); 
boolean isInteractive = pm.isInteractive(); 
mAPI.sleep(); 
mAPI.wakeup();

9. Navigation bar and status bar

Navigation bar: VISIBLE (visible); INVISIBLE (invisible but can be swiped up); ALWAYS_INVISIBLE (invisible and cannot be swiped out).

mAPI.setNavigationBarVisibility(SUNTEK.NavigationBarVisibility.ALWAYS_INVISIBLE); 
SUNTEK.NavigationBarVisibility v = mAPI.getNavigationBarVisibility(); 
if (v == SUNTEK.NavigationBarVisibility.ALWAYS_INVISIBLE) { 
mAPI.setNavigationBarVisibility(SUNTEK.NavigationBarVisibility.VISIBLE); 
}

A restart is required for the settings in the drop-down status bar to take effect.

mAPI.setExtendStatusBarVisibility(SUNTEK.ExtendStatusBarVisibility.VISIBLE); 
SUNTEK.ExtendStatusBarVisibility sv = mAPI.getExtendStatusBarVisibility();

10. Root privileges

if (!mAPI.isRoot()) { 
if (mAPI.enableRoot(true)) { 
Log.d(TAG, "Successful root"); 
} 
}

11. Ethernet

IpConfig ipConfig = mAPI.getIpConfig(); 
String ip = ipConfig.ip; 
String mask = ipConfig.mask; 
String gateway = ipConfig.gateway; 
ArrayList<String> dnsList = ipConfig.dnsList; 

mAPI.setStaticIp(new IpConfig("192.168.1.2", "255.255.255.0", "192.168.8.1", 
Arrays.asList("192.168.8.1"))); 
mAPI.setDhcpIp(); 

SUNTEK.IpMode mode = mAPI.getIpMode(); // STATIC / DHCP 
boolean open = mAPI.isEthernetOpen(); 
mAPI.setEthernetState(true);

12. System Time

Calendar calendar = Calendar.getInstance(Locale.getDefault()); 
calendar.set(2021, 10, 28, 6, 0, 0); // Months start from 0 
mAPI.setSystemTime(calendar.getTimeInMillis());

Scheduled power on/off: type=1 only takes effect once; type=3 cycles weekly, requires resetting after factory reset or flashing the firmware.

int type = 3; 
int[] weekdays = {1, 3, 4, 5}; // Monday, Wednesday, Thursday, Friday 
int[] poweroff = {20, 30}; 
int[] poweron = {8, 35}; 
mAPI.setPowerOnOffAlarmCycle(type, weekdays, poweroff, poweron); 
mAPI.cancelPowerOnOffAlarm(); 

boolean netTime = mAPI.isEnableNetworkProvidedTime(); 
mAPI.setEnableNetworkProvidedTime(true);

13. Hardware Module

13.1 GPIO

GPIO is used for I/O, relays, fill lights, USB power supply, cash drawer, doorbell, etc. The actual ports supported by the board are specified in the hardware datasheet. Constants are defined in SUNTEK. The listener function polls approximately once per second, making it unsuitable for detecting key presses less than one second in duration.

| constant | value | meaning |
| --- | --- | --- |
| SUNTEK.GPIO_1 … GPIO_15 | 0～14 | General I/O |
| SUNTEK.LIGHT_RED / GREEN / BLUE / WHITE | 15-18 | Fill light |
| SUNTEK.LIGHT_INFRARED | 19 | USB camera infrared fill light |
| SUNTEK.RELAY | 20 | relay |
| SUNTEK.BELL | twenty one | doorbell |
| SUNTEK.CASHBOX_0 / CASHBOX_1 | 22 / 23 | Cash Box |
| SUNTEK.LIGHT_BOARD_BLUE / RED / YELLOW | 24 / 28 / 30 | Onboard indicator lights |
| SUNTEK.USB_1 / USB_2 / USB_3 | 25 / 26 / 27 | USB powered |
| SUNTEK.LIGHT_CAMERA_EXTERNAL_INFRARED | 29 | MIPI camera infrared fill light |

if (mAPI.getGpioState(SUNTEK.GPIO_1) == SUNTEK.GpioState.HIGH) { } 
mAPI.setGpioState(SUNTEK.GPIO_1, SUNTEK.GpioState.HIGH); 
SUNTEK.GpioMode mode = mAPI.getGpioMode(SUNTEK.GPIO_1); 
mAPI.setGpioMode(SUNTEK.GPIO_1, SUNTEK.GpioMode.INPUT); 

SUNTEK.GpioListenerCallback cb = new SUNTEK.GpioListenerCallback() { 
public void onChanged(int index, int oldValue, int newValue) { } 
}; 
mAPI.listenGpio(SUNTEK.GPIO_1, cb); 
mAPI.unlistenGpio(SUNTEK.GPIO_1, cb);

13.2 LED brightness

mAPI.setLightBrightness(SUNTEK.LIGHT_RED, 204); // 0～255, 204 ≈ 80%

13.3 Wiegand

SUNTEK.WiegandMode wm = mAPI.readWiegandMode(); 
mAPI.writeWiegandMode(SUNTEK.WiegandMode.INPUT); 

// Synchronous blocking read, please put it in a background thread 
long code = mAPI.readWiegand(); // Asynchronous read: For continuous reads, you must register 

SUNTEK.WiegandCallback 
again in onSuccess / onFailure wcb = new SUNTEK.WiegandCallback() { 
public void onSuccess(long code) { mAPI.readWiegandAsyn(this); } 
public void onFailure() { mAPI.readWiegandAsyn(this); } 
}; 
mAPI.readWiegandAsyn(wcb); // Callback in worker thread 

mAPI.writeWiegand(SUNTEK.WiegandFormat.FORMAT_26, 12345678);

14. Reference Code

14.1 Cash Box

First lower it by 500ms, then raise it.

CompletableFuture.runAsync(() -> { 
mAPI.setGpioState(SUNTEK.CASHBOX_0, SUNTEK.GpioState.LOW); 
try { TimeUnit.MILLISECONDS.sleep(500); } catch (InterruptedException e) { } 
}).thenAccept(u -> { 
mAPI.setGpioState(SUNTEK.CASHBOX_0, SUNTEK.GpioState.HIGH); 
});

14.2 Executing the root command

boolean executeCmd(String content) throws IOException, InterruptedException { 
Process p = Runtime.getRuntime().exec("su"); 
PrintWriter pw = new PrintWriter(p.getOutputStream(), true); 
pw.println(content); 
pw.println("exit"); 
return p.waitFor() == 0; 
}

Appendix. Explanation